# Public-Building Vending Accessibility Verification Checklist

ADA Title II/III + 2010 ADA Standards Section 308/309 + VPAT 2.5 + annual audit

Public-building vending placements must satisfy 2010 ADA Standards for Accessible Design (Sections 305, 308, 309, 703) plus state-specific overlays (e.g., California Title 24, Texas TAS, Massachusetts 521 CMR). This checklist covers install verification, digital accessibility (Section 508 / WCAG 2.1 AA), and annual recertification. Operator should attest in writing and provide measurement documentation, VPAT 2.5, and a written remediation plan for any non-conforming items.

# 1. Reach range verification — 2010 ADA Standards Section 308

Reach range applies to every operable part: selection buttons, card reader, contactless tap zone, PIN pad, dispense flap, change-return lever, refund button. Measure with a calibrated tape and record to the procurement file.

☐  Unobstructed forward reach: highest operable part ≤ 48 in AFF and lowest operable part ≥ 15 in AFF (308.2.1).

☐  Obstructed forward reach: if obstruction depth ≤ 20 in, max reach 48 in; if 20-25 in, max reach 44 in (308.2.2).

☐  Unobstructed side reach: highest operable part ≤ 48 in AFF, lowest ≥ 15 in AFF (308.3.1).

☐  Obstructed side reach: if obstruction ≤ 10 in deep, max reach 48 in; if 10-24 in, max reach 46 in (308.3.2).

☐  Card reader + contactless tap zone + mobile-wallet target each independently meet reach range.

☐  Dispense bin opening within reach range and not below 15 in AFF.

# 2. Clear floor space + approach — Section 305

☐  30 in × 48 in minimum clear floor space at each machine face for forward or parallel approach (305.3).

☐  Clear space free of permanent obstructions (trash cans, A-frame signage, queue stanchions).

☐  Knee + toe clearance under any machine overhang per 306 where forward approach is used.

☐  Floor surface stable, firm, slip-resistant (302.1); carpet pile ≤ 1/2 in with secured edges (302.2).

☐  Accessible route to machine ≥ 36 in wide (404 / 403.5.1) with no abrupt level change > 1/4 in (303).

# 3. Operating force + dexterity — Section 309.4

☐  All operable parts operable with one hand and require ≤ 5 lbf to operate (309.4).

☐  No tight grasping, pinching, or twisting of the wrist required (recessed paddles vs round knobs).

☐  Selection buttons distinguishable by touch where the interface is visual-only (raised, recessed, or tactile).

☐  Coin/bill slot openings ≥ 1.25 in to accommodate limited dexterity.

☐  Refund / change-return mechanism operable without bending below 15 in AFF or reaching above 48 in.

# 4. Digital interface + Section 508 + WCAG 2.1 AA

Federal placements (GSA, VA, DoD, Smithsonian, federal courthouses) must conform to Section 508 Revised standards (36 CFR Part 1194) — which incorporates WCAG 2.1 Level A and AA. State agencies often adopt equivalent standards (e.g., California Gov Code 7405).

☐  Operator provides VPAT 2.5 (Voluntary Product Accessibility Template) covering kiosk firmware, payment terminal, and mobile app.

☐  Touchscreen interfaces support audio output via headphone jack (508 / WCAG 1.1.1 + 1.4.4).

☐  Text contrast ratio ≥ 4.5:1 for normal text, ≥ 3:1 for large text (WCAG 1.4.3).

☐  Mobile app and refund portal conform to WCAG 2.1 AA; operator provides accessibility statement URL.

☐  Card reader supports speech output (e.g., Verifone or Ingenico models with headphone jack) at federal placements.

# 5. Signage, labeling, allergen disclosure

☐  Product labels visible from clear floor space without glare; LED lighting ≥ 200 lux on selection face.

☐  Allergen labeling per FALCPA (21 U.S.C. 343) on each SKU; allergen summary on machine face.

☐  Pricing display characters ≥ 5/8 in with ≥ 70% contrast ratio (Section 703.2).

☐  Tactile braille pricing labels at federal placements and where state code requires (e.g., 521 CMR 41.0).

☐  Nutrition disclosure compliant with FDA calorie-labeling rule (21 CFR 101.8) for chains with 20+ machines.

# 6. Verification documentation + annual recertification

☐  Operator written ADA attestation signed at install and at each annual recertification.

☐  Photo + measurement log retained in procurement file (calibrated tape, measurements to 1/4 in).

☐  VPAT 2.5 dated within 12 months and on file.

☐  Site visit verification scheduled annually; non-conformance triggers 30-day remediation plan.

☐  Facility accessibility coordinator + ADA coordinator (per 28 CFR 35.107 for Title II entities) copied on attestation.

☐  Procurement file retains documentation for contract term plus 5 years (or per agency record retention).

☐  Tenant / public complaint procedure published; complaints logged + responded to within 15 business days.

Informational only — not legal advice. Verify federal placement requirements with the facility ADA coordinator and the contracting officer; state and local jurisdictions may add requirements above the federal floor. LetUsVending is equipped to coordinate operator selection and verification documentation for public-building placements.