# 28-Point ADA Campus Vending Audit Checklist

ADA Title III + Section 308 + WCAG 2.1 AA audit for higher-education vending programs

Use this checklist at install and at annual recertification for every vending machine and AI cooler on a campus. Each item maps to a specific ADA Title III, 2010 ADA Standards (Section 308/309), or Section 508 / WCAG 2.1 AA requirement applicable to public-accommodation higher-education facilities.

# Reach range — 2010 ADA Standards Section 308

Operable parts include selection buttons, payment readers, dispense doors, and any touchscreen interface. Reach-range failures are the most common ADA non-conformance on US campuses.

☐  1. Highest operable part ≤ 48 inches above finished floor — unobstructed forward reach (Section 308.2.1).

☐  2. Lowest operable part ≥ 15 inches above finished floor (Section 308.2.1).

☐  3. Obstructed forward reach — obstruction depth ≤ 20 inches, highest point ≤ 48 inches (Section 308.2.2).

☐  4. Side reach — unobstructed ≤ 48 inches; obstructed (depth ≤ 10 inches) ≤ 48 inches; obstructed (depth 10-24 inches) ≤ 46 inches (Section 308.3).

# Clear floor space + circulation — Section 305

☐  5. 30 inch × 48 inch clear floor space provided for parallel or forward approach to every operable part.

☐  6. Floor surface stable, firm, and slip-resistant per Section 302.1.

☐  7. If carpeted, pile height ≤ ½ inch with secured edges (Section 302.2).

☐  8. Clear floor space not in a primary circulation path that conflicts with traffic flow.

# Operating force — Section 309.4

☐  9. Operable parts function with one hand and require ≤ 5 lbf to operate.

☐  10. No tight grasping, pinching, or twisting of the wrist required.

☐  11. Dispense door opens with ≤ 5 lbf and does not require pinching.

☐  12. Card reader insertion / contactless tap functional at all reach ranges in scope.

# Visual + signage accessibility — Section 703 + WCAG 2.1 AA

☐  13. Pricing display contrast ratio ≥ 70% (Section 703.5.1) — independently measured, not vendor-attested.

☐  14. Character height ≥ 5/8 inch where pricing is wall-posted or machine-face.

☐  15. Product labels visible from the clear floor space — no glare, no obstruction.

☐  16. Allergen labels per FALCPA on every SKU; allergen summary visible on machine face.

☐  17. Tactile braille pricing labels where state code or campus access office requires.

# Digital interface — Section 508 + WCAG 2.1 AA (touchscreens, kiosks, apps)

AI cooler and micro-market touchscreen kiosks must meet WCAG 2.1 Level AA. Campus access offices increasingly require a VPAT (Voluntary Product Accessibility Template) before procurement.

☐  18. Touchscreen has an audio-output mode or paired mobile app for blind / low-vision users (WCAG 1.1.1, 1.4.5).

☐  19. Touch targets ≥ 44 px × 44 px (WCAG 2.5.5 AAA — campuses often require AAA on this criterion).

☐  20. Text contrast on screen ≥ 4.5:1 for body text, ≥ 3:1 for large text (WCAG 1.4.3).

☐  21. No content relies solely on color to convey meaning (WCAG 1.4.1).

☐  22. Operator provides current VPAT on request; revision date within the last 12 months.

# Payment + refund accessibility

☐  23. Cashless option available so users without coin/bill handling capability can complete transactions.

☐  24. Refund flow accessible via app, SMS, or phone — not solely a wall-mounted touchscreen out of reach.

☐  25. Mobile wallet (Apple Pay / Google Pay) supported as an alternative to card insertion.

# Documentation + recertification

☐  26. Operator provides written ADA attestation at install and annually.

☐  27. Annual site-visit verification with photographs and measurements documented; campus access office copied.

☐  28. Procurement file retains documentation for the duration of the contract plus 7 years (Higher Education Act recordkeeping).

This checklist is informational and not legal advice. Coordinate with the campus ADA / Section 504 coordinator and disability resource office. State and local jurisdictions may add requirements above the federal floor.