# Outdoor Vending Placement Site Survey Checklist

Equipment rating, power, overhead protection, climate, anchoring, and cellular connectivity

Use this checklist before quoting an outdoor vending placement. Outdoor placements multiply the failure modes of an indoor install — equipment must be rated for outdoor use, electrical must meet NEC outdoor branch-circuit requirements, and the site must support reliable telemetry. Mark each item as Verified, Accepted with mitigation, or Reject before scheduling install.

# 1. Equipment rating and enclosure

☐  Machine listed for outdoor use by manufacturer (outdoor-rated SKU or outdoor enclosure model).

☐  Cabinet UL 751 (commercial refrigerator) or comparable listing for any cold/refrigerated unit; NEMA 3R or NEMA 4 enclosure rating for the electrical compartment.

☐  Operating temperature window stated by manufacturer; typical outdoor units rated -10°F to 110°F ambient.

☐  Vandal-resistant glass / polycarbonate front per manufacturer outdoor spec (typical thickness ≥ ½ inch).

☐  Cash box and dispense bin locking mechanisms designed for outdoor use (corrosion-resistant tumblers or electronic locks).

# 2. Power and electrical — NEC outdoor branch-circuit requirements

☐  Dedicated 20A 120V circuit (or 240V where required), wired to a GFCI receptacle per NEC 210.8(B) for outdoor commercial locations.

☐  Receptacle in a weatherproof in-use cover (NEMA WD-6 / UL 514D 'extra duty' rating) regardless of whether a plug is currently engaged.

☐  Conductor and conduit rated for outdoor / wet locations (UF cable, RMC, or LFMC per NEC Article 300).

☐  Surge protective device (SPD) rated for the branch circuit; outdoor lightning exposure significantly raises risk.

☐  Voltage drop ≤ 3% at maximum running load — verify with a clamp meter under compressor start.

# 3. Overhead protection and ground conditions

☐  Permanent overhead cover (roof / awning / canopy) extends at least 24 inches beyond the machine footprint on all sides where rain or sun exposure is direct.

☐  Slope or drainage adequate so water does not pool at the machine base; machine sits on a curb, pad, or platform above the surrounding finished grade.

☐  Distance from sprinkler heads, downspouts, and roof drainage outlets at least 36 inches.

☐  Direct sun exposure on the cabinet face limited; south- or west-facing placements may exceed manufacturer ambient rating in summer.

# 4. Climate and seasonal operation

☐  Refrigerated SKU placement: ambient temperature in coldest month ≥ manufacturer minimum (typically 32-40°F).

☐  Snack SKU placement: chocolate and confection products require ambient ≤ 75°F for shelf integrity.

☐  Humidity exposure: outdoor humidity > 90% with poor ventilation accelerates corrosion; specify a corrosion-resistant cabinet finish.

☐  Wind exposure: anchoring schedule sized for the local design wind speed per ASCE 7 (typically 90-130 mph nominal, 3-second gust).

# 5. Anchoring, security, and ADA

☐  Machine anchored to concrete pad or building structure per manufacturer outdoor anchoring spec (typical: 4 × ½ inch concrete anchor bolts to 4 inch slab).

☐  Bollards or curb stops protect the machine front from vehicle impact where parking, drive lanes, or pedestrian traffic is within 8 feet.

☐  Lighting: at least 10 foot-candles at the machine face during operating hours per IESNA outdoor commercial recommendation.

☐  ADA Title III + Section 308 reach range, clear floor space, and operating force met at the outdoor location (same standards apply outdoors).

☐  CCTV or operator-monitored security camera covers the machine face where the site is public-facing or operates overnight.

# 6. Cellular connectivity and telemetry

☐  Cellular signal strength at machine location ≥ -100 dBm RSRP (LTE) measured by operator at survey, with the carrier the telemetry modem uses.

☐  Fallback connectivity option (alternate carrier SIM, Wi-Fi bridge to building network) defined if primary signal is marginal.

☐  Telemetry heartbeat configured ≤ 15 minutes; offline alert SLA ≤ 30 minutes.

☐  Cashless acceptance latency verified at site — outdoor installs occasionally fail tap-to-pay if the modem cannot maintain a session.

This checklist is informational and not engineering advice. Outdoor placements should be reviewed by a licensed electrician and a code official for NEC, ASCE 7 wind, and local jurisdiction requirements. LetUsVending can connect you with operators equipped for outdoor placements — request the outdoor capability matrix to compare candidates.