# Venue Eco-Vending Framework

Six-dimension spec, certified-SKU curation, ESG data feed integration, operator MSA template

Use this framework to specify and operate an eco-vending program at an entertainment venue (theater, arena, stadium, theme park, convention center). Each section is capability-framed and references EPA ENERGY STAR, AHRI 1410 (vending machine efficiency), Green Seal + USDA BioPreferred + BPI compostability certifications, FTC Green Guides for environmental claims, and FDA Food Code 2022 + FALCPA for the food-safety baseline. Operator capabilities should be confirmed in writing before contract execution.

# 1. Six-dimension eco specification

Eco-vending is not a single attribute. The six dimensions below cover the lifecycle of a vending unit and the products inside.

| Dimension | Spec target | Reference standard |
| --- | --- | --- |
| 1. Energy efficiency | ENERGY STAR certified vending equipment; ≤ 1,800 kWh/year per unit at standard test conditions | EPA ENERGY STAR + AHRI 1410 |
| 2. Refrigerant + cold-chain | HFC-free refrigerant (R-290, R-744 / CO2); GWP < 150 | SNAP-approved per EPA SNAP + EU F-Gas |
| 3. Packaging | ≥ 80% of facings in recyclable, compostable (BPI), or refillable packaging | BPI + USDA BioPreferred + How2Recycle labels |
| 4. Product certifications | Defined certified-SKU floor (Fair Trade, Rainforest Alliance, USDA Organic, Non-GMO, B Corp) | Per-certification body + FTC Green Guides |
| 5. Waste + recycling | Co-located 3-bin waste sort (landfill / recycle / compost); operator removes its own packaging waste | EPA WasteWise + venue policy |
| 6. ESG reporting | Quarterly data feed: kWh, refrigerant inventory, packaging mix, certified-SKU %, waste tonnage diverted | GRI 305 + SASB Food Retailers |

# 2. Certified-SKU curation

Define a certified-SKU floor in the contract — for example, ≥ 60% of facings carry at least one third-party sustainability certification. Operator should attest in writing and provide quarterly chain-of-custody documentation.

Fair Trade USA / Fairtrade International: Coffee, tea, cocoa, sugar, banana. Operator provides FLO-CERT or Fair Trade USA chain-of-custody certificate.

Rainforest Alliance: Coffee, tea, cocoa, banana. Operator provides RA certificate + traceability ID.

USDA Organic: Snack, beverage, dairy. Operator provides NOP (National Organic Program) certificate.

Non-GMO Project Verified: Snack, beverage, dairy. Operator provides Non-GMO Project verification ID.

B Corp: Brand-level certification. Operator provides current B Corp certification of supplier.

BPI compostable + How2Recycle: Packaging certifications applied to disposables, cups, lids, utensils, snack film.

# 3. Equipment specification — ENERGY STAR + refrigerant

ENERGY STAR refrigerated beverage vending: AMS, Crane, Royal, Vendo, USI with ENERGY STAR certification; ≤ 1,800 kWh/year for full-size beverage vending per ENERGY STAR v3.1 spec.

ENERGY STAR refrigerated combo + snack: Combo + snack vending ENERGY STAR-certified or labeled where category is covered.

Refrigerant: R-290 (propane) or R-744 (CO2) preferred (GWP < 5); R-513A or R-454B acceptable transitional refrigerants (GWP < 600). Avoid R-404A or R-134a.

LED lighting: 10+ year life LED interior + face lighting; dimmable + occupancy-sensor-equipped where feasible.

Vending Misers / occupancy controls: USA Technologies VendingMiser, Cantaloupe SmartPower, or comparable PIR + thermostat sensors; typical 30-40% energy reduction at low-traffic placements.

# 4. Operator capability checks

☐  Operator should attest to ENERGY STAR-certified equipment for all refrigerated units and provide AHRI 1410 + ENERGY STAR documentation on file.

☐  Operator should attest to HFC-free or low-GWP refrigerant on all new placements; transition plan for legacy units.

☐  Operator should attest to ≥ X% certified-SKU floor (define in contract) with quarterly chain-of-custody documentation.

☐  Operator should attest to FTC Green Guides compliance for any environmental claim made on the machine face, packaging, or marketing material.

☐  Operator should attest to packaging-waste removal cadence + diversion-rate reporting (target ≥ 60% diversion).

☐  Operator should attest to ESG data feed delivery (kWh, refrigerant inventory, packaging mix, certified %, tonnage diverted) on a quarterly cadence.

☐  Operator should attest to NSF/ANSI 7 + 25 + FDA Food Code + FALCPA baselines unchanged by the eco overlay.

# 5. ESG data feed integration

Where the venue publishes a sustainability report (GRI, SASB, CDP, TCFD-aligned), operator-side vending data should plug in. Define the data feed below.

Energy: Per-machine kWh per month from operator's telemetry; aggregated and reported quarterly. Maps to GRI 302-1.

Refrigerant: Refrigerant inventory + leak-report log; tied to Scope 1 emissions calculation. Maps to GRI 305-1.

Scope 3 — purchased goods: Per-SKU embodied carbon estimate where supplier provides it; aggregated quarterly. Maps to GRI 305-3 + SASB FB-FR-110a.1.

Waste: Operator-removed packaging tonnage + diversion-rate %. Maps to GRI 306-2 + SASB FB-FR-150a.1.

Certified-product %: Per-quarter % of facings with each certification body. Maps to SASB FB-FR-430a.2.

Format: Operator delivers CSV or API feed monthly to venue ESG team; integration option with Workiva, Sphera, Persefoni, or comparable ESG platform.

# 6. Operator MSA — eco-overlay provisions

Layer the eco-overlay onto a standard vending Master Services Agreement. Specific provisions below.

Sustainability rep + warranty: Operator reps + warrants ENERGY STAR + refrigerant + certified-SKU compliance; periodic re-attestation required (annual).

Audit rights: Venue may audit operator's ESG data feed accuracy annually; discrepancy > 5% triggers operator-paid third-party audit.

Certified-SKU floor: Operator attests to maintaining ≥ X% certified-SKU facings; quarterly compliance audit; sustained drop > 5 pts below floor triggers cure period.

FTC Green Guides compliance: Operator reps + warrants no false or unsubstantiated environmental claims; indemnifies venue for any FTC enforcement action arising from operator claims.

Equipment refresh + lifecycle: Operator commits to equipment refresh schedule (typically 7-10 years) with each refresh meeting or exceeding the ENERGY STAR + refrigerant standard at time of refresh.

Termination + transition: Standard termination clauses apply; eco-overlay obligations survive termination for past performance periods + ongoing reporting obligations.

# 7. Reporting + quarterly review

☐  Monthly: per-machine kWh, transactions, basket, refund + downtime, ESG-flag SKU velocity.

☐  Quarterly: certified-SKU % audit + chain-of-custody documentation review.

☐  Quarterly: ESG data feed delivery + reconciliation with venue's ESG platform.

☐  Quarterly: packaging-waste tonnage + diversion rate + audit photos.

☐  Annual: full ENERGY STAR + refrigerant + certification recertification; equipment refresh planning.

☐  Annual: contract review — certified-SKU floor reset, sustainability target update aligned with venue's net-zero or 1.5°C-aligned trajectory.

This framework is informational and not legal, environmental-compliance, or ESG-reporting advice. Environmental claims must comply with the FTC Green Guides (16 CFR Part 260) and applicable state false-advertising laws. ESG reporting frameworks evolve (GRI, SASB → ISSB, CDP, CSRD); confirm current standards with the venue's sustainability team.