# Hotel Vending ADA Placement Checklist

Reach range, clear floor space, signage, and audible / visual feedback verification

Use this checklist before signing a hotel vending contract and at annual recertification. Each item maps to a specific ADA Title III or Section 308 / 305 / 309.4 / 703 requirement. Operator should be able to attest in writing and provide documentation on request.

# 1. Reach range — ADA Section 308

Reach range applies to operable parts (selection buttons, payment readers, dispense doors, refund release). Verify both forward and side reach where the device permits parallel approach.

☐  Highest operable part ≤ 48 in AFF for unobstructed forward reach (Section 308.2.1).

☐  Lowest operable part ≥ 15 in AFF (Section 308.2.1).

☐  Obstructed forward reach: depth of obstruction ≤ 20 in and highest operable part ≤ 48 in (Section 308.2.2); if depth 20-25 in, highest operable part ≤ 44 in.

☐  Side reach: highest operable part ≤ 48 in unobstructed (Section 308.3.1); over obstruction up to 10 in deep, ≤ 48 in; 10-24 in obstruction, ≤ 46 in (Section 308.3.2).

☐  Card reader, contactless reader, PIN pad, and dispense flap each meet reach range and operate with one hand without tight grasping, pinching, or twisting (Section 309.4).

# 2. Clear floor space — ADA Section 305

☐  30 × 48 in clear floor space provided for parallel or forward approach to operable parts.

☐  Clear floor space free of permanent obstructions and not within a major circulation path that creates traffic conflict.

☐  Carpet pile ≤ ½ in with secured edges per Section 302.2.

☐  Vending alcove door clear width ≥ 32 in per Section 404.2.3; door operating force ≤ 5 lbf per Section 404.2.9.

# 3. Signage and high-contrast pricing — ADA Section 703

☐  Pricing display character height ≥ 5/8 in per Section 703.5.5 where pricing serves as building signage.

☐  Contrast ratio ≥ 70% between characters and background per Section 703.5.1.

☐  Tactile braille pricing labels at federal placements or where state code requires (Section 703.3 + 703.4).

☐  Allergen icons supplement text-only allergen lists; FALCPA disclosure on every SKU.

☐  Pricing visible from the 30 × 48 in clear floor space; no glare-inducing fluorescent reflection on the display.

# 4. Audible and visual feedback — ADA Section 309 / 707

☐  Selection feedback: audible tone or click confirms selection.

☐  Visual feedback: display confirms selection text large enough to read from the clear floor space (typical character ≥ ½ in).

☐  Error states (out of stock, payment declined, refund pending) clearly indicated by both audible and visual cues — not visual-only.

☐  Dispense complete confirmation by both audible and visual cue.

# 5. Verification and documentation

☐  Operator provides written ADA attestation at install and annually thereafter.

☐  Operator provides VPAT (Voluntary Product Accessibility Template) for digital interfaces — card reader, kiosk, app.

☐  Site-visit verification scheduled at install and annually; photos and measurements documented.

☐  Hotel accessibility coordinator copied on all attestation documents and retains records for contract term + 5 years.

☐  Complaint log: guest accessibility concerns recorded, root-cause analyzed, remediation owner and due date assigned.

This checklist is informational and not legal advice. ADA Title III applies to hotels as places of public accommodation; state and local jurisdictions may add requirements above the federal floor. Coordinate the final review with counsel. LetUsVending can connect you with operators equipped to attest to each item — request the hotel capability matrix to compare candidates.