# 28-Point First Vending Machine Inspection Checklist

Pre-purchase inspection, paperwork verification, and condition scoring

Use this checklist before you wire funds for a used or refurbished vending machine. Bring it printed to the seller's facility, score each line, and walk away from machines scoring below 70 / 100. References cite NAMA Industry Standard 19 (vending machine vending) and FDA Food Code 2022 Part 4.

# Cabinet + structural condition (1-5)

The cabinet is the most expensive component to refurbish. Surface rust, frame damage, or door-seal failure typically signals a machine past its economic life.

☐  1. Exterior paint — no major chips, scratches, or rust. Touch-ups acceptable; rust-through is not.

☐  2. Door alignment — gasket seals continuously when closed; no gaps when paper is inserted around the perimeter (refrigerated units).

☐  3. Hinges + latch — no excessive play, latch engages firmly without forcing.

☐  4. Coin-mech door + bill-validator door — keys operate freely, locks engaged and not drilled-out / replaced with substitutes.

☐  5. Cabinet feet / leveling — all four feet present and adjustable. Cabinet sits flat without rocking.

# Refrigeration + cold-chain (refrigerated units only) (6-10)

FDA Food Code 2022 §3-501.16 requires refrigerated potentially hazardous foods to be held at 41°F or below. A failing compressor or evaporator is the #1 reason used refrigerated machines are unprofitable.

☐  6. Compressor — runs without excessive noise (no metal-on-metal grinding), cycles on within 30 seconds of cabinet temp rising.

☐  7. Internal cabinet temperature — measured with an independent probe, 33-40°F throughout duty cycle.

☐  8. Condenser coil — clean (or pre-cleaned), fins not damaged, fan operates without bearing whine.

☐  9. Evaporator — frost pattern even, no excessive ice buildup, drain pan empty and drain line clear.

☐  10. Refrigerant type — verify R-134a, R-290, or R-744 (CO2). Pre-2010 machines may use R-22 which is EPA-phased-out and expensive to recharge.

# Payment systems (11-15)

PCI-DSS v4.0 (effective March 2024) places liability for card-data breaches on the operator. Card readers must be on the current MDB-compliant generation; legacy readers will not pass PCI audit.

☐  11. Card reader — Nayax, Cantaloupe, USA Technologies, or 365 Retail Markets; PCI-DSS v4.0 compliant; cellular SIM active or transferable.

☐  12. Bill validator — MEI/CPI/JCM model accepting current US series; rejection rate ≤ 5% on test bills.

☐  13. Coin mech — accepts current US coinage; coin return functional; payout tubes loaded for change.

☐  14. MDB harness — clean wiring, all connectors seated, no aftermarket splices or electrical tape repairs.

☐  15. Cashless transaction test — operator performs a $1 test sale and confirms it lands in the merchant account dashboard.

# Selection mechanism + dispense (16-20)

☐  16. Spirals / coils — uniform spacing, no kinks or off-axis rotation; spirals can be reshaped or replaced individually.

☐  17. Drop sensor / dispense detection — every selection completes with the iVend or shelf-detect circuit confirming product fell.

☐  18. Selection keypad — every digit / letter responsive, no missing or worn keycaps.

☐  19. Door interlock — machine inhibits sales when door is open.

☐  20. Test vend — operator runs a free test vend on every row; no jams, no double drops.

# Electrical + safety (21-24)

UL 541 is the safety standard for refrigerated vending; UL 751 for non-refrigerated. A missing or removed UL label is a red flag — it usually means the cabinet was rebuilt outside the original spec.

☐  21. UL 541 or UL 751 label present on cabinet (refrigerated vs non-refrigerated).

☐  22. Power cord intact, no nicks or repairs; matches the cabinet rating (15A or 20A NEMA 5-15P / 5-20P).

☐  23. Internal lighting — LED preferred; no broken ballasts or exposed wiring.

☐  24. Ground continuity — verified with a multimeter (chassis-to-ground).

# Paperwork + documentation (25-28)

☐  25. Bill of sale — signed by the seller, lists make/model/serial number, sale price, and 'as-is' or warranty terms.

☐  26. Title / origin — for ex-route machines, a delivery receipt or original purchase invoice from the OEM/dealer.

☐  27. Service history — at minimum, the prior operator's service-call summary for the last 12 months.

☐  28. Owner's manual + planogram template — should accompany the machine; otherwise download from the OEM (Crane, AMS, Royal, Seaga, USI/Wittern, Federal Industries).

Score each item 0 (fail), 2 (acceptable), or 4 (excellent). Machines scoring below 70 / 112 are not worth the negotiated purchase price; renegotiate or walk away. This checklist is informational, not legal advice.