# Warehouse Vending Install Verification Checklist

Site survey + electrical scope + cellular + 3-shift planogram + ANSI / OSHA placement

Warehouses and distribution centers require an install scope that mid-rise office HVAC + 9-to-5 placements never face: ambient temperature swings, 24/7 operation, three shifts, forklift traffic, and OSHA egress + emergency-eyewash + storage-rack interaction. This checklist gives the facility manager, EHS lead, and procurement officer a contract-ready pre-install survey and a 30/60/90 day startup audit. Operator should attest in writing to each item.

# 1. Pre-install site survey

Conduct the site survey with operator, facility manager, and EHS lead present. Document every measurement with photos. Retain in procurement file.

☐  Floor load: machine + product weight ~700-1,000 lbs; verify floor rating per ASCE 7-22 (typical warehouse slab ≥ 250 psf — far above requirement).

☐  Doorway clearance to placement: ≥ 36 in wide, no permanent threshold > 1/2 in (ADA 303.3); double-doors where machine width > 30 in.

☐  Forklift / pallet-jack delivery path documented — operator typically delivers via 27-30 in width machine on pallet jack.

☐  Ambient temperature range at placement (record min/max over 7 days).

☐  Humidity range at placement (record min/max over 7 days).

☐  Cellular signal: ≥ -100 dBm RSRP (LTE Cat-1) or equivalent verified with cellular meter — not phone bars.

☐  Proximity to break room, restroom, time clock; walking distance < 200 ft from production floor.

# 2. Electrical scope of work

☐  Dedicated 120V / 20A circuit per machine (NEC 210.23(B)(2)); not shared with break-room appliances.

☐  GFCI protection where placement is within 6 ft of sink, wash bay, or eyewash station (NEC 210.8).

☐  Receptacle within 6 ft of machine placement (NEC 400.8 — no extension cords).

☐  Refrigerated machine: minimum 12 in clearance from rear coil to wall (manufacturer spec); confirm with Royal Vendors, Crane Merchandising Systems, AMS, or Vendo install guide.

☐  Coffee / hot beverage equipment: 30A circuit minimum where pulled-water; consider 240V split for high-draw espresso.

☐  Lockout / tagout point identified for maintenance access (29 CFR 1910.147).

☐  Surge suppression / dedicated transient voltage surge suppressor where industrial loads are switched on the same panel.

# 3. Connectivity + telemetry

☐  Cellular preferred for industrial sites — separates payment + telemetry from facility production network.

☐  If facility Wi-Fi: VLAN isolated from production network; PSK rotated quarterly; segmented per PCI DSS v4.0.1 Req 1.2.6.

☐  Telemetry vendor: Cantaloupe Seed, Nayax MoMa, 365 Retail Markets ADM, AWM Smart Shelf, or equivalent — heartbeat ≤ 15 min.

☐  Operator dashboard read-access for facility manager + EHS + procurement.

☐  Service alert SLA: <24 hr response for outage, <24 hr resolution for revenue-critical / cold-chain fault.

# 4. ANSI / OSHA placement requirements

☐  Egress + corridor width per IBC 1005 + NFPA 101 maintained after machine install (typical 44 in minimum; verify locally).

☐  Machine not blocking eyewash station, emergency shower, fire extinguisher, fire pull, or first-aid cabinet (ANSI Z358.1, NFPA 10).

☐  Forklift traffic separation: machine 18+ in from drive aisle striping; bollards installed where impact risk elevated (ANSI MH16.1).

☐  Anchor / tip-over restraint installed — Consumer Product Safety Commission has documented fatal tip-over incidents.

☐  Floor signage / striping demarcates pedestrian access path (OSHA 1910.144 color-coding).

☐  Adequate lighting on machine face (≥ 200 lux) — many warehouse zones below this baseline.

# 5. Three-shift planogram baseline

Warehouses run shift 1 (typically 6 AM-2 PM), shift 2 (2 PM-10 PM), shift 3 (10 PM-6 AM). Per-shift velocity will differ; planogram should be telemetry-tuned at 30 and 90 days post-install.

Shift 1 (6 AM-2 PM): Breakfast bars, fresh fruit, yogurt, RTD coffee, water, balanced lunch combo. Higher fresh / cold-chain weighting.

Shift 2 (2 PM-10 PM): Snacks, sodas, energy drinks, lunch leftovers, heat-and-eat entrées where micro-market.

Shift 3 (10 PM-6 AM): Caffeine beverages (energy + cold brew), hot drinks, comfort snacks, calorie-dense meal replacements.

PPE-friendly grip: SKU packaging openable with glove on (avoid micro-perforation pull tabs at heavy-glove sites).

Hydration emphasis: Water 30-40% of beverage facings at hot / heavy-PPE sites; electrolyte SKUs where heat-illness risk elevated (OSHA heat NEP).

# 6. 30 / 60 / 90 day startup audit

1. Day 30

Telemetry-derived per-shift velocity vs install projection; refund volume; payment failure rate; first planogram refresh.

2. Day 60

Stockout incidents + resolution; full QBR draft; ANSI / OSHA placement walk-through with EHS lead.

3. Day 90

Formal QBR with operator: uptime SLA adherence, planogram v2, capture-rate vs ADP target, refund rate, action items + owners.

# 7. Operator MSA + insurance verification

☐  Operator carries $2M general liability minimum; warehouse/EHS-elevated sites often $5M.

☐  Workers comp + auto liability current; certificate of insurance names facility as additional insured.

☐  Background-checked service techs per facility policy; site induction + PPE briefing completed.

☐  After-hours service procedure aligned with facility security / lockout policy.

☐  Operator commits to 5-business-day stockout resolution + 24-hour outage response in writing.

Informational only — not engineering or safety advice. Verify NEC, IBC, NFPA, ANSI, and OSHA requirements with facility EHS lead and local AHJ. LetUsVending is equipped to coordinate warehouse-experienced operators with 24/7 SLAs and three-shift planogram tuning.