# Hotel Vending Install Checklist

Placement zones, brand finish, PMS integration, per-alcove electrical, and scheduling

Use this checklist when installing vending machines in a hotel — full-service, limited-service, extended-stay, or boutique. Hotel vending installs differ from typical commercial sites in three ways: brand finish requirements (cabinet wrap, signage), PMS / room-charge integration, and scheduling around guest occupancy. Mark each item Verified, Acceptable with mitigation, or Reject before scheduling install.

# 1. Placement zones and machine selection

| Zone | Recommended machine | Common SKU mix |
| --- | --- | --- |
| Per-floor alcove | Beverage + snack combo or beverage-only | Water, soda, candy, chips, snack bars |
| Lobby off-corridor | Combo or micro-market kiosk | Grab-and-go meals, toiletries, OTC, snacks, beverages |
| Fitness center | Beverage + protein combo | Water, electrolyte, protein bars, towels |
| Pool deck (outdoor-rated) | Beverage-only outdoor-rated | Water, soda, sunscreen, accessories |
| Business center / lounge | Snack + coffee combo | Coffee, water, snack bars, breakfast items |

# 2. Brand finish and cabinet wrap

☐  Cabinet wrap or paint specified to brand standards (cabinet color, side panel graphic, header signage).

☐  Brand-flag logo placement approved by hotel's brand-standards manager and operator's print vendor.

☐  Wrap material rated for indoor commercial use (typical 3M Controltac or equivalent vinyl) with 3-5 year UV/wear life.

☐  Wrap installation scheduled during install or pre-staging — not retrofitted after the cabinet is on the floor.

☐  Brand-standards review form signed off by hotel general manager and operator account manager.

# 3. PMS integration and room-charge billing

☐  PMS integration feasibility confirmed: Oracle OPERA, Mews, Cloudbeds, Marriott FOSSE, Hilton OnQ, or comparable property-management system.

☐  Room-charge interface: vending machine reads guest room key (HID card) or accepts mobile-wallet linked to folio.

☐  Daily folio reconciliation between operator telemetry and PMS revenue ledger; mismatch tolerance ≤ 1% by dollar volume.

☐  Refund flow integrated with PMS so room-charged refunds appear on the folio rather than as a separate operator-side credit.

☐  PCI DSS scope: card reader and any embedded PMS integration each maintain compliant scope (current PCI DSS version).

# 4. Per-alcove electrical and plumbing

☐  Each alcove provided with a dedicated 20A 120V circuit per NEC 210.23; refrigerated combo or hot-food machines may require 240V.

☐  GFCI per NEC 210.8(B) for placements within 6 ft of a sink or in wet locations; weather-resistant in-use cover for outdoor / pool-deck.

☐  Voltage drop ≤ 3% at running load per NEC 210.19(A) Informational Note 4 — long alcove runs from the panel may require upsizing conductors.

☐  Plumbing: coffee or water machines need a ¼-in cold-water line with shutoff valve, pressure 30-80 psi, backflow protection per local plumbing code.

☐  Sprinkler deflector clearance ≥ 18 in per NFPA 13 Section 8.6.5; HVAC diffuser placement does not blow directly onto cabinet face.

# 5. Scheduling and guest impact

☐  Install scheduled during low-occupancy window (typical 10 AM - 3 PM weekdays); avoid weekend and holiday peaks.

☐  Elevator reservation booked with front desk so machine delivery does not interfere with check-in/out traffic.

☐  Floor protection plan (Masonite or plywood track) documented to prevent damage to lobby and corridor flooring.

☐  Acoustic considerations: cabinet placement away from guest-room walls; refrigeration cycle noise ≤ 55 dB at 1 m per typical hotel guest-area limit.

☐  Sign-off walk-through with hotel engineering, front-desk manager, and operator account manager at install completion.

# 6. ADA and accessibility verification

☐  ADA Section 308 reach range (≤ 48 in / ≥ 15 in AFF), Section 305 clear floor space (30 × 48 in), Section 309.4 operating force (≤ 5 lbf).

☐  Section 703 character height ≥ 5/8 in and contrast ratio ≥ 70% for pricing display.

☐  Audible + visual feedback on selection, payment, dispense, and refund states.

☐  Operator provides written ADA attestation at install and annually thereafter; VPAT for digital interfaces.

This checklist is informational and not engineering, accessibility-law, or PCI-compliance advice. Electrical and plumbing work must be performed by licensed professionals per NEC and local plumbing code; PMS integration and PCI scoping should be reviewed with the hotel's IT and PCI assessor. LetUsVending can connect you with operators equipped for hotel placements — request the hotel capability matrix to compare candidates.