# Interactive Vending Screen Verification Framework

Capability checklist + MDM + ADA / Section 508 + nutrition disclosure + privacy

Touchscreen-equipped vending machines (Crane Merchandising Systems Streamware-enabled, AMS Touch, USA Technologies / Cantaloupe ePort Engage, Vendekin, smart coolers from Cooler Screens, AAEON, Selecta) introduce new procurement scrutiny: mobile-device management (MDM) for content / firmware, ADA + Section 508 conformance on the interface, FDA calorie-disclosure rules for menu-board-style displays, and US state privacy law obligations where the screen captures imagery or behavior. This framework gives facilities, procurement, and IT a contract-ready verification checklist.

# 1. Hardware capability

☐  Touchscreen ≥ 21.5 in for full menu-board UX; ≥ 7 in acceptable as selector display.

☐  Capacitive (PCAP) touch with glove-mode support where industrial / cold-room placement.

☐  Brightness ≥ 700 nits where ambient lighting ≥ 500 lux; sun-readable ≥ 1,500 nits at semi-outdoor placement.

☐  IP-rated bezel (IP54+) where wash-down or splash risk; tempered + anti-vandal cover glass.

☐  Headphone jack (3.5 mm) for audio output per Section 508 / WCAG 2.1 AA.

☐  On-screen keyboard + dwell-click support for limited-mobility users.

# 2. MDM + content management

Operator must run a managed-device infrastructure equivalent to an enterprise MDM (Microsoft Intune, Jamf, SOTI, VMware Workspace ONE). Without MDM the screen becomes a managed-content liability — stale calorie labels, expired promotions, unauthorized content.

☐  Operator MDM platform documented; firmware + content push remote-managed.

☐  Content workflow: planogram changes pushed to screen within 24 hours of operator system update.

☐  Calorie + allergen content tied to SKU master; updates push automatically with planogram.

☐  Content approval gate: facility / SFA / wellness committee has review-and-block right on creative content.

☐  Audit log: content + firmware change events retained 12 months minimum.

☐  Kiosk mode locked; OS-level access prevented (Android Enterprise / Windows Assigned Access).

# 3. ADA + Section 508 + WCAG 2.1 AA

☐  VPAT 2.5 (Voluntary Product Accessibility Template) dated within 12 months on file.

☐  Touchscreen highest operable target ≤ 48 in AFF (2010 ADA Standards 308.2.1).

☐  Operating force ≤ 5 lbf; usable with one hand and no tight grasping (Section 309.4).

☐  Audio output via 3.5 mm jack supports screen-reader-equivalent narration of menu + price + allergen.

☐  Text contrast ratio ≥ 4.5:1 (WCAG 1.4.3); ≥ 3:1 for large text.

☐  Font size minimum equivalent to 14-pt at typical viewing distance; magnification supported.

☐  Touch target ≥ 9.6 mm (WCAG 2.5.5 enhanced); spacing prevents adjacent-selection error.

☐  Dwell-click / sticky-keys / single-hand operation modes supported.

# 4. Nutrition + calorie disclosure

☐  FDA Menu Labeling Rule (21 CFR 101.8) applies to chains with 20+ machines under common ownership — verify scope.

☐  Calorie display per selection (and per item where multi-serve) — same display prominence as price.

☐  Allergen labeling per FALCPA (21 U.S.C. 343); Big 9 allergens including sesame (FASTER Act 2021).

☐  Smart Snacks SKU flags at K-12 placements per 7 CFR 210.11.

☐  Nutrition Facts panel viewable on-screen or via QR linking to authoritative source (SKU master).

☐  State / city overlays verified: New York City Health Code Section 81.50 (calorie posting), California Prop 65 (acrylamide / heavy metals).

# 5. Privacy + data — US state law overlay

Smart coolers + camera-equipped screens may collect imagery, demographic inference, dwell time, or behavioral data — triggering CCPA/CPRA (California), VCDPA (Virginia), CPA (Colorado), CTDPA (Connecticut), UCPA (Utah), and 15+ other state laws active or in-effect through 2026.

☐  Operator data-collection inventory documented: imagery (yes/no), facial analysis (yes/no), dwell time, transaction telemetry.

☐  Privacy notice posted at machine + on operator website per CCPA §1798.100; mirror disclosures for VCDPA / CPA / CTDPA / UCPA.

☐  If facial analysis: BIPA (Illinois 740 ILCS 14) consent obtained where applicable; Texas CUBI (Tex. Bus. & Com. Code 503.001) and Washington (RCW 19.375) overlays verified.

☐  If targeted advertising: opt-out mechanism + Global Privacy Control honored.

☐  Data retention schedule documented; minimum-necessary principle applied.

☐  Vendor data processing agreement (DPA) in place; sub-processor list maintained.

# 6. Security + payment integration

☐  Payment via PCI PTS v6+ approved terminal; PCI DSS v4.0.1 AOC on file (effective 31 March 2025).

☐  Touchscreen UX never captures PAN; payment device handles entry + tokenization.

☐  P2PE v3.1 or strong tokenization; no PAN on operator host.

☐  Screen-to-network: TLS 1.2 minimum (TLS 1.3 preferred); SSL / TLS 1.0/1.1 disabled (NIST SP 800-52 Rev. 2).

☐  Firmware update process signed; downgrade prevention enforced.

☐  Vulnerability scanning quarterly; SBOM available on request.

# 7. Contract + operational

☐  MSA references VPAT 2.5, PCI DSS v4.0.1, FDA 21 CFR 101.8, FALCPA, applicable state privacy laws.

☐  Quarterly content + privacy + accessibility review built into QBR.

☐  Operator commits to <24 hour content removal SLA for non-compliant or inappropriate content.

☐  Operator carries cyber liability insurance ≥ $1M; data-breach notification cooperation.

☐  Sunset / decommission procedure: data deleted per retention schedule on contract end.

Informational only — not legal advice. Verify accessibility, FDA, FTC, and state privacy law applicability with counsel and the facility ADA / wellness coordinator. LetUsVending is equipped to coordinate operators with VPAT-documented touchscreen platforms and content-management SLAs.