# Venue Vending Sound + Light Experience RFP Template

Six-category design, sensory accommodation protocol, OSHA compliance, and operator scoring

Use this RFP template to solicit vending bids where machines incorporate sound, light, motion, or interactive sensory elements (illuminated wraps, sound triggers, motion-activated digital sidekicks, AR/QR integrations). The template specifies the six experience categories, the sensory-accommodation protocol required for accessibility, and the OSHA compliance posture for staff working near amplified-sound and high-illumination installations.

# 1. Project overview

Venue + event mix: (stadium / arena / theater / convention / experiential brand activation — specify capacity, event types)

Machine count + placements: Specify zones and indicate which categories of sensory experience apply to each placement. Capability framing — describe environment, not anchor venues.

Contract term: 2-4 year initial with annual capability refresh (sensory-content + firmware).

Power + data: Operator specifies power draw, data connection (cellular vs. venue WAN), and any audio routing requirements.

# 2. Six experience categories

| # | Category | Specification |
| --- | --- | --- |
| 1 | Illuminated front face | LED-backlit translucent vinyl with diffuser; UL 60950 or UL 8750 listed; ENERGY STAR-class lighting; CCT specified per venue (typically 3000K-4000K). Brightness adjustable for circadian and audience-comfort tuning. |
| 2 | Edge / accent lighting | RGB or programmable warm-white edge lighting on machine perimeter; dimmable; DMX or proprietary CMS-controllable for event-themed scenes. |
| 3 | Sound triggers | Sub-70 dBA peak at 3 ft per ANSI/ASA S12.2 for ambient placements; sub-85 dBA short-burst on transaction confirmation. Mute / accessibility-mode required (see Section 3). |
| 4 | Motion-activated digital sidekick | 21"-32" commercial-grade display; motion sensor triggers contextual content when audience approaches. UL 60950 listed. |
| 5 | AR / QR / NFC integration | Tap or scan triggers branded landing page, AR overlay, or sweepstakes. NFC ISO/IEC 14443 standard; tag durability rated for 100,000+ taps. |
| 6 | Interactive cabinet (touch / gesture) | Touchscreen front overlay or gesture-camera selection. Touchscreen complies with Section 308 reach range; gesture interface offers tactile-button fallback per ADA 309.4. |

# 3. Sensory-accommodation protocol

Sensory installations must be accessible to audiences with sensory-processing needs (autism spectrum, photosensitive epilepsy, hearing aids, low vision). Operator must offer the following modes by default.

☐  Mute mode: any sound trigger can be silenced at the machine (physical button or app toggle) and silenced venue-wide via CMS for sensory-friendly performances.

☐  Reduced-motion mode: motion-activated content can be set to static during sensory-friendly events.

☐  Photosensitive-epilepsy compliance: no flashing content at 3-30 Hz frequency per W3C WCAG 2.2 Success Criterion 2.3.1 and ITU-R BT.1702.

☐  Lighting dimming: edge + accent lighting dimmable to 10% of peak; pre-show + sensory-friendly mode profile preset.

☐  Hearing-aid T-coil compatibility for any speech audio (FCC Section 68.317 hearing-aid-compatible) where audio is informational rather than ambient.

☐  Captioning for any video / spoken content on digital sidekicks; WCAG 2.2 Level AA captioning standard.

☐  Operator publishes a sensory-accommodation summary that venue accessibility coordinator can distribute to audiences in advance.

# 4. OSHA + staff compliance

Noise exposure (OSHA 29 CFR 1910.95): Staff working > 8 hours within 5 ft of machines with sustained > 85 dBA require hearing-conservation program. Operator confirms ambient levels per ANSI/ASA S12.2.

Lighting safety: LED + edge-lighting installations must comply with IEC 62471 photobiological-safety limits; no Class 2 or higher hazard exposure to staff or audience.

Electrical (NFPA 70 / NEC): All wiring run in metal conduit or NEC-rated cable assemblies; GFCI on any wet-zone or exterior placement; UL 60950 / UL 62368 listed power supplies.

Egress + clearances: Machines + signage do not obstruct egress per IBC Chapter 10; minimum 36-inch clearance maintained in front of machine for ADA Section 305 clear floor space.

Staff training: Operator's on-site staff complete sensory-mode operation training; documented before first event.

# 5. Scoring matrix

| Criterion | Weight |
| --- | --- |
| Six-category execution capability (Section 2) | 25% |
| Sensory-accommodation protocol (Section 3) | 20% |
| OSHA + compliance posture (Section 4) | 10% |
| CMS + content-management platform | 10% |
| Hardware durability + warranty | 10% |
| Reporting + analytics (dwell, scan, engagement) | 10% |
| Pricing transparency | 10% |
| Insurance + bonding | 5% |

# 6. Required attachments

☐  Portfolio: 3+ comparable sensory installations (capability framing — describe install scale and category mix, not anchor venues).

☐  UL / IEC / ANSI compliance documentation per Section 4.

☐  Sensory-accommodation summary sheet venue can distribute pre-event.

☐  CMS platform demo (Stratacache, BrightSign, Enplug, or comparable).

☐  Sample dwell + engagement analytics report.

☐  Insurance certificate, $5M GL minimum given experiential-equipment risk.

☐  References: 3 venue marketing / F&B / accessibility contacts (last 24 months).

# 7. Submission instructions

Format: PDF response, with appendix for sample sensory profile, CMS demo video link, and compliance documentation.

Due date: (specify — typically 30-45 days given complexity)

Pre-bid demo: Operator demonstrates a single working machine with all six categories, or provides a recorded demo if travel infeasible.

Award: (specify — typically 45-60 days)

Informational template — engage venue procurement, accessibility coordinator, and risk counsel before issue. Sensory-accommodation requirements are governed by ADA Title III and venue policy; some jurisdictions exceed federal minimums.