# School Vending Security + Anti-Vandalism Checklist

Hardware hardening + placement + programming + telemetry SLA + incident response

K-12 and higher-ed placements face theft, vandalism, tampering, and tip-over risk distinct from office or industrial sites. This checklist gives the SFA director, school resource officer, and facilities team a defensible baseline — hardware, placement, programming, telemetry, and incident response. Map each item to the operator MSA so accountability is contractual, not informal.

# 1. Hardware hardening

☐  Cabinet steel gauge ≥ 18 ga (preferably 16 ga) for front + sides; pry-resistant edges.

☐  T-handle lock with rotating disc-detainer (Medeco, Abloy Protec2, or equivalent) — keyed unique to district.

☐  Anti-tip-over kit (NFPA-aligned floor anchor or wall tether) installed on every machine — Consumer Product Safety Commission has documented fatal tip-over incidents in school vending.

☐  Polycarbonate (not glass) selection window where impact risk elevated; minimum 1/4 in thickness.

☐  Bill validator + coin mech with anti-stringing + anti-spoon mechanisms (Coinco, MEI / CashFlow models).

☐  Cashbox / stacker on separate lock from cabinet door (dual-lock cash zone).

☐  External LED illumination on machine face — passive deterrent, also serves placement-visibility audit.

# 2. Placement security audit

☐  Machine within CCTV camera field-of-view; camera resolution ≥ 1080p, retention ≥ 30 days.

☐  Placement avoids dead-end alcoves and unmonitored exterior alcoves accessible after hours.

☐  Placement on accessible route but not directly adjacent to exterior egress door (reduces grab-and-run exit).

☐  Adjacent walls + floor able to bear machine weight (typical 600-900 lbs loaded) + tip-over anchor load.

☐  Egress / fire code clearances maintained (NFPA 101 / IBC 1004) — machine does not narrow corridor below code minimum.

☐  After-hours access policy: building lock-down schedule covers machine placements; key control logged.

# 3. Programming + payment hardening

☐  Bill validator firmware updated to current revision; counterfeit detection enabled (UV + magnetic + IR).

☐  Card reader on PCI PTS v6+ approved devices list; PCI DSS v4.0.1 AOC dated within 12 months.

☐  P2PE (PCI P2PE v3.1) or strong tokenization enabled — no clear-text PAN on operator host.

☐  Cashless-only programming option for high-risk placements (eliminates cash incentive).

☐  Refund + service mode access requires operator credential (PIN + telemetry-logged).

☐  Maximum coin/bill payout per refund event capped — limits payout fraud exposure.

# 4. Telemetry + alerting SLA

Telemetry is the primary detection layer for tampering, door-open events, payment failure, and unusual velocity. Tie each alert to a contractual SLA in the operator MSA.

Heartbeat: Cellular modem heartbeat ≤ 15 minutes; offline > 30 minutes triggers operator alert + 24-hour resolution SLA.

Door-open alert: Any unscheduled door-open event triggers SMS + email to operator + district facility manager within 5 minutes.

Tamper alert: Tilt sensor + impact sensor (where supported) triggers tamper alert; review within 1 hour.

Payment failure rate: Failure rate > 3% on rolling 24-hour window triggers operator investigation within 4 hours.

Velocity anomaly: Out-of-pattern SKU velocity (refund spike, single-SKU drain) flagged within 24 hours.

Dashboard read-access: SFA director + school resource officer have telemetry read-access (Cantaloupe Seed, Nayax MoMa, 365 Retail Markets ADM, or equivalent).

# 5. Incident response procedure

1. Detection

Telemetry alert OR student / staff report OR scheduled walk-through finding. Time-stamp the report immediately.

2. Containment

Operator dispatches within service SLA. Take machine offline (cashless disabled, vend disabled) until inspected. CCTV footage from incident window preserved by district.

3. Investigation

Operator + facilities + (where appropriate) SRO conduct walk-through within 24 hours. Photo + measurement log; identify hardware, payment, or placement root cause.

4. Remediation

Hardware repair or replacement within 5 business days; programming + telemetry adjustments; placement change if root cause is placement.

5. Reporting

Incident report to SFA director + facilities + (where applicable) law enforcement. Quarterly incident-trend report at QBR.

# 6. Quarterly + annual review

☐  Quarterly incident-trend report from operator (per-machine, per-incident-type).

☐  Annual penetration walk-through with SRO + facilities + operator service tech.

☐  Annual telemetry SLA adherence report (uptime, response time, resolution time).

☐  Annual cabinet key audit — keys reissued if compromise suspected or staff turnover.

☐  Insurance + bonding: operator carries $2M GL minimum; theft / vandalism coverage rider verified.

Informational only — not legal or security advice. Coordinate placement decisions with district facilities, school resource officer, and risk management. LetUsVending is equipped to coordinate operators with hardened cabinets, telemetry SLAs, and per-quarter incident reporting for K-12 and higher-ed placements.