# Vending Machine Size + Space Requirement Checklist

Dimensions + maneuvering clearance + doorway + floor load + ADA approach

Before a placement is approved, verify the machine fits — not only at its final position, but along the entire delivery path from receiving to that position. Standard manufacturer dimensions, ADA approach clearances (28 CFR Part 36 + 2010 ADA Standards), and floor-load minimums (ASCE 7-22) are summarized here. Use during site survey, operator selection, and on the day of delivery.

# 1. Typical machine dimensions (manufacturer reference)

| Machine class | Width | Depth | Height | Empty weight |
| --- | --- | --- | --- | --- |
| Snack (5-wide) | 37-41 in | 33-36 in | 72-75 in | 550-650 lbs |
| Snack (6-wide) | 42-46 in | 33-36 in | 72-75 in | 600-700 lbs |
| Beverage (glass-front) | 37-41 in | 33-36 in | 72-79 in | 650-750 lbs |
| Beverage (stack-loaded) | 37-41 in | 32-36 in | 72-79 in | 700-800 lbs |
| Combo (snack + beverage) | 39-41 in | 36-38 in | 72-75 in | 650-750 lbs |
| Frozen / refrigerated fresh food | 39-41 in | 36-41 in | 72-75 in | 750-900 lbs |
| Coffee / espresso (counter-top) | 16-22 in | 20-26 in | 30-36 in | 60-150 lbs |
| Coffee / espresso (floor) | 24-32 in | 26-32 in | 65-72 in | 250-450 lbs |
| AI / smart cooler | 26-39 in | 31-38 in | 75-82 in | 550-800 lbs |

# 2. Loaded weight + floor load

☐  Snack: ~700 lbs loaded; beverage: ~900-1,000 lbs loaded; frozen: ~1,000-1,100 lbs loaded.

☐  Commercial floor minimum live load 50 psf — any commercial slab clears requirement.

☐  Above-grade wood floor: verify ≥ 40 psf per IBC Table 1607.1; reinforce or relocate if sub-rated.

☐  Raised access floor (data center, modern office): verify panel + pedestal rating ≥ 1,250 lb point load.

☐  Anti-tip-over restraint anchored to floor or wall — CPSC has documented fatal tip-over incidents.

# 3. Delivery path clearances

☐  Doorway clear width ≥ 36 in (ADA 404.2.3); permanent threshold ≤ 1/2 in (ADA 303.3).

☐  Corridor turn: ≥ 60 in for 180-degree turn with machine on hand truck; pivot point measured at machine center.

☐  Elevator inside clear ≥ 42 in × 51 in (typical passenger code minimum 6 ft × 4 ft 3 in); freight elevator strongly preferred.

☐  Elevator door clear width ≥ 36 in; verify cab rating ≥ 2,500 lb where machine + hand truck combined.

☐  Stairs: machine cannot be delivered up stairs over 2 risers without piano dolly + 2-person crew + insurance rider.

☐  Ramp grade ≤ 1:12 (ADA 405.2) where ADA path; ≤ 1:8 acceptable for delivery-only path with appropriate crew.

# 4. Placement clearances + ventilation

☐  Rear clearance: 3-6 in refrigerated (compressor venting); confirm manufacturer cut sheet (Royal Vendors, Crane Merchandising Systems, AMS, Vendo).

☐  Side clearance: 1-2 in minimum each side for service access; 6+ in preferred for side-load planogram refresh.

☐  Top clearance: 4 in minimum for refrigerated condenser intake on top-mount units.

☐  Service door swing: ≥ 30 in clear forward of machine to fully open + load.

☐  Ambient temp at placement 50-90°F; refrigerated machines derate above 90°F.

# 5. ADA clear floor space + approach — 2010 ADA Standards

☐  30 in × 48 in clear floor space at machine face (Section 305.3) for forward or parallel approach.

☐  Accessible route ≥ 36 in wide leads to clear floor space (Section 403.5.1; 32 in minimum at point of obstruction).

☐  Reach range: highest operable part ≤ 48 in AFF; lowest ≥ 15 in AFF (Section 308).

☐  Knee + toe clearance under any machine overhang per Section 306 where forward approach used.

☐  Clear floor space free of permanent obstructions (trash cans, signage, queue stanchions).

# 6. Egress + code clearances

☐  Corridor width after machine install ≥ code minimum (IBC 1020 / NFPA 101 — typically 44 in commercial).

☐  Machine does not block fire extinguisher (NFPA 10), eyewash (ANSI Z358.1), AED, fire pull, or first-aid cabinet.

☐  Egress path clearance per IBC 1005 maintained at full occupancy.

☐  Means-of-egress signage + lighting unobstructed.

☐  AHJ + fire marshal sign-off where placement is in egress path or assembly occupancy.

# 7. Pre-delivery sign-off

☐  Facility manager confirms placement + delivery path measurements (photo + dimensions logged).

☐  Receiving + dock access coordinated; appointment scheduled.

☐  Operator confirms manufacturer + model + dimensions in delivery confirmation.

☐  Security + badging arranged for install crew.

☐  Floor protection (Masonite or equivalent) prepared along delivery path on finished floor.

Informational only — not engineering or code advice. Verify IBC, NFPA, ADA, and manufacturer dimensions with the AHJ + operator + manufacturer cut sheet. LetUsVending is equipped to coordinate operator site surveys, delivery-path verification, and ADA-compliant placements.