# Event-Season Vending Maintenance Framework

Cadence, dark-window calendar, surge kit, validator service schedule

Use this framework to operate a vending program at venues with concentrated event seasons (stadiums, arenas, festival sites, fairgrounds, ski resorts, water parks). Each section is capability-framed and references NSF/ANSI 7 + 25, AHRI 1410, FDA Food Code 2022 § 3-501.16, MDB (Multi-Drop Bus) refrigeration + payment interfaces, EVA-DTS telemetry, and ADA Section 308. Operator capability should be confirmed in writing before contract execution.

# 1. Maintenance cadence template

Event-season placements wear faster than steady-state placements. The cadence below assumes high event-day load + dark periods between seasons.

| Task | Pre-season | In-season | Post-season |
| --- | --- | --- | --- |
| Coil + condenser clean | Yes (T-30) | Every 60 days or per telemetry alert | Yes (T+14) |
| Refrigerant + pressure check | Yes (T-30) | If telemetry flag | Yes (T+14) |
| Bill validator + coin mech service | Yes (T-30) | Every 30 days | Inspect |
| Payment reader (contactless / EMV) | Yes (T-30) | Firmware audit every 30 days | Yes (T+14) |
| Telemetry + connectivity test | Yes (T-30) | Daily heartbeat monitored | Yes (T+14) |
| LED + signage inspection | Yes (T-30) | Pre-each event | Yes |
| Gasket + door alignment | Yes (T-30) | Every 60 days | Yes (T+14) |
| ADA + accessibility audit | Yes (T-30) | Annual | — |
| Pest control inspection | Yes (T-30) | Every 30 days | Yes |

# 2. Dark-window calendar

Schedule major preventive maintenance in dark windows (no events) to avoid event-day disruption. Sample calendars below — replace with venue-specific schedule.

| Venue type | Dark windows | Major-PM target window | Refresh / refurb window |
| --- | --- | --- | --- |
| NFL stadium | Feb - Aug (training-camp aside) | Mar - May | Apr - May |
| MLB stadium | Nov - Mar | Dec - Feb | Jan - Feb |
| College football stadium | Dec - Aug | Apr - Jul | May - Jul |
| Concert arena (general) | Limited; typically 2-3 wk windows | Per-week scheduled overnight | Annual 1-2 wk shutdown |
| Festival site / fairgrounds | Off-season month per venue | Off-season | Off-season |
| Ski resort | Apr - Oct | May - Aug | Jun - Aug |
| Water park | Nov - Mar (northern) | Dec - Feb | Jan - Feb |

# 3. Surge kit list — event-day go-kit

Operator service crew carries a surge kit on event days. Goal: ≤ 30-min mean-time-to-restore for common failures.

Bill validator + coin mech spares: Pre-tested validator (MEI VN 2300, Coinco BV5500, MARS AE2832) + coin mech (Coinco 9302L, MEI CF7000); MDB harness.

Payment reader spares: Contactless / EMV reader (Cantaloupe ePort, Nayax VPOS Touch, USA Tech ePort) pre-paired with cellular SIM.

Refrigeration parts: Common condenser fans, compressor relays, evaporator fans, gaskets; charged refrigerant cylinder (R-290, R-744 per OEM).

Tools + diagnostics: MDB harness tester, EVA-DTS reader, multimeter, manifold gauges, leak detector, clamp meter, label printer.

Restock SKUs: Top-20 SKUs in surge quantity (2x normal day); cold-chain ice + insulated transport.

Comms: 2-way radio paired with venue ops, cellphone with venue's incident-management app, printed SLA + contact tree.

# 4. Validator + payment service schedule

Bill validators and payment readers are the highest-failure-rate components in event-day venues. Schedule below targets ≥ 99% transaction-acceptance rate.

Bill validator: Clean lens + belt every 30 days; full firmware update every 90 days; full bench test pre-season + mid-season.

Coin mech (where present): Clean every 30 days; jam check pre-event; coin tube inventory pre-event.

Contactless / EMV reader: Firmware audit every 30 days; antenna + battery check every 60 days; full PCI-aligned attestation annual.

MDB cabling + harness: Inspect for chafing + corrosion every 60 days; replace at any signs of intermittent drop.

Cellular modem + SIM: RSSI test pre-event; SIM swap every 24 months or per carrier sunset notice; failover to dual-SIM where possible.

# 5. Operator capability checks

☐  Operator should attest to dedicated event-season service crew with ≤ 30-min mean-time-to-restore for Tier-1 failures during event hours.

☐  Operator should attest to NSF/ANSI 7 + 25 + AHRI 1410 certifications on refrigerated equipment and FDA Food Code 3-501.16 cold-chain compliance.

☐  Operator should attest to EVA-DTS-compliant telemetry on every event-placement unit with ≤ 15-minute heartbeat and 30-minute alert threshold for offline status.

☐  Operator should attest to PCI-DSS 4.0 cashless payment + bill validator service per OEM cadence + ≥ 99% transaction-acceptance rate.

☐  Operator should attest to dark-window major-PM scheduling + event-day surge kit on site for every event-day shift.

☐  Operator should provide $5M general liability + $5M umbrella + product liability ≥ $2M + additional-insured endorsement to venue (event-venue baseline).

# 6. Event-day operations workflow

1. T-72: telemetry + inventory check

Operator pulls telemetry for all event-placement units; confirms no offline status; pre-positions restock load.

2. T-24: pre-event restock + readiness check

Full restock of all event-day units; validator + reader test; cold-chain check; surge kit verified on site.

3. T-4: final walk-through

Service crew on-site walk-through; final inventory build; comms check with venue ops.

4. T-0 to event-end: event monitoring

Live telemetry monitored; ≤ 30-min mean-time-to-restore for any Tier-1 failure; mid-event top-off as needed.

5. T+0 to T+4: post-event restock + reconciliation

Mid-night restock; reconciliation of cash, cashless, refunds; cold-chain end-of-event report.

6. T+24: post-event review

Uptime, transactions, refund volume, service tickets, kit replenishment. Drives next-event prep.

# 7. Reporting + quarterly review

☐  Per-event: uptime, transactions, basket, refund volume, validator + reader acceptance rate, service tickets.

☐  Monthly: PM completion, telemetry health, refrigeration performance, planogram refresh.

☐  Quarterly: dark-window PM schedule update + capital refresh planning for any units approaching end-of-life.

☐  Annual: full lifecycle review — PCI + insurance + food safety recertification; equipment refresh proposals.

This framework is informational and not engineering, legal, or food-safety advice. Maintenance scope, refrigerant handling, and PCI scope each require qualified technicians and current OEM documentation. Confirm operator capability through reference checks and written attestation.